# Cross-Reference Analysis: Battery Recycling Equipment Research

## Consolidated Findings from Multiple Sources

## 1. Executive Overview

This document synthesizes findings from three key sources:

- Executive Strategic Recommendation - Cost-optimized hybrid approach (Chinese + Western equipment)

- PhD Research Buyer’s Guide - CAPEX-first analysis for 1-5 t/h scale

- Equipment Recommendations - Detailed technical specifications

The PhD research document provides critical additional insights that refine and expand upon the previous recommendations.

## 2. Key New Insights from PhD Research

### 2.1 ANDRITZ/Duesenfeld: The Game-Changer

| Aspect | Previous Documents | PhD Research Update |
| --- | --- | --- |
| Process | Standard mechanical recycling | Low-temperature vacuum (<80°C, ≤300 hPa) |
| HF Gas | Requires scrubbing | Eliminates HF formation - no scrubbing needed |
| Equipment | Requires stainless steel | Standard carbon steel sufficient |
| Electrolyte | Destroyed in drying | Recovered at >99.5% purity - revenue stream |
| CAPEX | 40-60% savings vs BHS | 50% savings vs conventional Western plants |
| Recycling Rate | 72% (mechanical) | 91% at cell level |

Critical Finding: The Duesenfeld process fundamentally changes the economics by eliminating three major cost centers: HF gas scrubbing, stainless steel equipment, and continuous nitrogen consumption.

### 2.2 Additional Western Suppliers Identified

| Supplier | Scale | Key Feature | Status |
| --- | --- | --- | --- |
| Erdwich (Germany) | 50 kg - 5 t/h | Wet + dry shredding options | Targets exact 1-5 t/h range |
| ELDAN (Denmark) | 4 t/h | ELDAN TOWER integrated system | Recently launched |
| MTB Recycling (France) | 1-4 t/h | Containerized Drycell Box | Pilot running since Sept 2023 |
| Primobius/SMS (Germany) | 0.3 t/h | Mercedes reference, wet shredding | Lower throughput |

### 2.3 Chinese Suppliers: Enhanced Analysis

| Supplier | CAPEX | Verified Installations | Key Risk |
| --- | --- | --- | --- |
| Genox | $500K-$1.2M | Partnerships with Sicon (Germany), Genox USA/UK | No ATEX certification |
| Xingmao | $300K-$800K | 1.5 t/h USA, 1 t/h Singapore, 2 t/h South Korea | After-sales logistics |
| Henan Doing | $70K-$350K | Transparent pricing | Lowest tier quality |

PhD Assessment: Xingmao provides strongest evidence of international viability with verified installations in USA, Singapore, and South Korea.

## 3. Process Flow: Modernized Recommendations

### Traditional 10-Step Process (Outdated)

- Discharge → 2. Pre-sort → 3. Primary Shred → 4. Secondary Crush → 5. Fine Milling → 6. Screening → 7. Magnetic Sep. → 8. Eddy Current → 9. Air Classification → 10. Collection

### PhD-Recommended 6-Step Process (CAPEX-Optimized)

- Deep electrical discharge with energy recovery

- Integrated shredding tower under N₂ (eliminates primary + secondary as separate steps)

- Vacuum drying at <80°C with electrolyte recovery (Duesenfeld-type)

- Screening at ~1mm cut point

- Separation train: magnetic → eddy current → air classification

- Black mass collection (cyclone + baghouse)

Key Changes: - Integrated shredding towers (ANDRITZ, ELDAN, BCA) achieve ≤15mm in single unit - Fine milling often eliminated if shredding achieves adequate liberation - Drying before separation improves downstream efficiency

## 4. CAPEX Comparison: All Sources

| Source | Capacity | CAPEX | Notes |
| --- | --- | --- | --- |
| BHS-Sonthofen | 1-15 kt/yr | €8-20M ($9-22M) | 3+ proven plants, >97% recovery |
| ANDRITZ/Duesenfeld | 2 t/h | Est. €5-12M | 50% less than BHS |
| Western Alternatives | 1-5 t/h | €3-8M | Erdwich, ELDAN, MTB |
| Chinese (Genox) | 1-3 t/h | $500K-$1.2M | European partnerships |
| Chinese (Xingmao) | 1-2 t/h | $300K-$800K | Verified international installs |
| Chinese (Henan Doing) | 500-3000 kg/h | $70K-$350K | Budget option |

## 5. Safety Systems: Critical Insights

### PhD Research Key Findings

| System | PhD Assessment | Implication |
| --- | --- | --- |
| ATEX Certification | Critical gap for Chinese equipment | Black mass dust = Zone 20/21/22 |
| CE Marking | Often self-declared, variable rigor | Third-party assessment rare |
| Wet Shredding | Eliminates inert gas needs | Reduces CAPEX significantly |
| Cryogenic | High OPEX, not recommended | Liquid nitrogen ongoing cost |

### Recommended Safety Architecture (Hybrid)

| Component | Recommended Source | Rationale |
| --- | --- | --- |
| Shredding (critical) | Western: BHS, ANDRITZ, Erdwich | ATEX compliance, insurance acceptance |
| N₂ Systems | Western: Generon | Proven reliability |
| Gas Detection | Western: Drager/Honeywell | ATEX certified |
| Fire Suppression | Western: Siemens/Tyco | Insurance requirement |
| Separation Equipment | Either Western or Chinese | Lower safety stakes |

## 6. Black Mass Quality: Realistic Expectations

### PhD Research Findings on Purity Limits

| Impurity | Typical Level | GB/T 45203-2024 Grade 1 Target |
| --- | --- | --- |
| Copper | 1-10% | <0.5% |
| Aluminum | 1-5% | <1% |
| Iron | 0.5-3% | <1% |

Key Insight: Meeting Grade 1 standards (Ni+Co>35%, Li>3.5%) is: - Achievable with production scrap (consistent chemistry) - Extremely challenging with mixed end-of-life batteries

### Feedstock Chemistry Impact

| Chemistry | Black Mass Value | Processing Challenge |
| --- | --- | --- |
| NMC | Highest (Ni 9-25%, Co 5-16%, Li 2-4%) | Standard processing |
| LFP | Lithium only | Iron creates separation issues |
| Mixed | Requires sorting | Must segregate NMC from LFP |

## 7. Consolidated Recommendations

### By Regulatory Environment

| Jurisdiction | Recommended Approach | Estimated CAPEX |
| --- | --- | --- |
| EU/ATEX-regulated | ANDRITZ/Duesenfeld pathway | €5-12M |
| Outside ATEX (SE Asia, Middle East, Africa) | Chinese from Genox/Xingmao | $500K-$1.5M |
| Aggressive CAPEX minimization | Wet shredding + Chinese separation | $300K-$800K |

### Universal Recommendations

- Invest disproportionately in screening - Multi-deck precision screens have highest marginal impact on purity

- Upstream chemistry sorting - XRF-based sorting separates NMC from LFP before processing

- Don’t eliminate fine milling entirely - Only if shredding tower achieves adequate liberation

- Drying before separation - PhD research confirms this improves downstream efficiency

## 8. ROI Analysis: Updated

### Consolidated from All Sources

| Scenario | Throughput | Annual Output | Revenue | Payback |
| --- | --- | --- | --- | --- |
| Conservative | 500 kg/hr | 300 tons | $900K | 36+ months |
| Expected | 1,000 kg/hr | 810-1,500 tons | $3-6M | 24-30 months |
| Optimistic | 2,000 kg/hr | 3,000+ tons | $12M+ | 18 months |

Critical Success Factors (All Sources Agree): 1. Feedstock security (2-3 year agreements) 2. Black mass offtake contracts 3. Safety system investment (non-negotiable) 4. Process optimization expertise

## 9. Summary: Cross-Document Consensus

| Factor | Executive Recommendation | PhD Research | Consensus |
| --- | --- | --- | --- |
| Best Western | BHS-Sonthofen | ANDRITZ/Duesenfeld | Both credible; Duesenfeld more cost-effective |
| Best Chinese | GEP ECOTECH | Xingmao | Xingmao has best international verification |
| Hybrid Approach | Recommended | Also recommended | Strong consensus |
| Safety Systems | Western mandatory | Western mandatory | Absolute consensus |
| CAPEX Range | $180K-350K | $300K-1.5M (Chinese) / €5-12M (Western) | Reasonable range |
| Process Steps | 10 stages | 6 stages (modernized) | PhD provides optimization |

## 10. Final Assessment

The PhD research document significantly enhances the strategic recommendation by:

- Validating the hybrid approach as a sound strategy

- Adding ANDRITZ/Duesenfeld as a compelling alternative to BHS

- Providing realistic CAPEX ranges for different supplier tiers

- Identifying process flow optimizations that reduce equipment count

- Clarifying impurity limits and quality expectations

- Highlighting ATEX as a critical decision factor

Bottom Line: For a CAPEX-first buyer at 1-5 t/h scale, the optimal path is: - EU/ATEX markets: ANDRITZ/Duesenfeld (€5-12M) - Other markets: Chinese from Xingmao or Genox ($500K-$1.5M) - Both paths: Maintain Western safety systems throughout